FATIMA JINNAH WOMEN UNIVERSITY

Submitted by:

Romaisa              2023-BCS-073

Sameen Noor      2023-BCS-077

Javeria Rahim     2023-BCS-046

Areeba Yasir       2023-BCS-016

Course: Computer Organization and Assembly language (LAB)

Section: BCS-4(B)

PROJECT REPORT

Project Title:

Password Verification System with LED Indicators Using EMU8086 and External Hardware Interface

Objective:

The main objective of this project is to build a password verification system using Assembly Language (EMU8086) that interfaces with external hardware to visually indicate the result of password input through LEDs on a breadboard.

Hardware Components Used:

- Breadboard – For connecting LEDs and circuit wiring.

- 3 LEDs (Red, Yellow, Green)

  - Red: Correct password

  - Yellow: Incorrect password

  - Green: Missing/Incomplete input

- Arduino Board (e.g., Arduino Nano) – Acts as a microcontroller to control the LEDs.

- USB to RS232 Converter Cable – For serial communication between PC and Arduino.

- MAX232 Converter – For TTL to RS232 voltage level conversion.

- Jumper Wires – For making necessary connections.

- Laptop with EMU8086 Assembler – To run and test the assembly program.

Software Used:

- EMU8086 – Emulator and assembler for writing and executing x86 assembly code.

- Arduino IDE – For uploading LED control code to Arduino

Working Principle:

- User Input: The user is prompted to enter a 6-character password via the keyboard.

- Password Check:

  - If the entered password matches the pre-set password (mypass), a success message is displayed, and the Red LED turns ON.

  - If the password does not match but is exactly 6 characters long, the Yellow LED turns ON indicating a wrong password.

  - If fewer than 6 characters are entered (incomplete input), the Green LED turns ON.

- Communication: The EMU8086 sends the result through the COM port to the Arduino, which then turns ON the corresponding LED.

The code uses basic DOS interrupt calls:

- INT 21h AH=01h: Read character from keyboard

- INT 21h AH=09h: Print string to console

Hardware Interfacing:

- EMU8086 doesn’t support direct hardware access, so serial communication is used.

- The EMU8086 sends ASCII data or signals through a COM port (virtual serial port).

- The Arduino listens on that serial port using Serial.read() in a loop.

- Based on the message ("Red", "Yellow", "Green"), Arduino turns ON the respective LED using digitalWrite().

Pin Configuration:

- Red LED → Arduino Pin 8

- Yellow LED → Arduino Pin 9

- Green LED → Arduino Pin 10

Limitations:

- EMU8086 cannot directly control GPIO pins; it depends on serial communication to pass instructions.

- Password is hard-coded and unencrypted.

- No real-time GUI feedback; only console messages and LED indicators.

Conclusion:

This project successfully demonstrates how low-level programming (Assembly Language) can interact with real-world hardware using serial communication. Despite EMU8086 being a legacy tool, it was used effectively to simulate a login system that triggers physical LED responses through an Arduino bridge. The project integrates both software and hardware concepts, showcasing basic embedded system principles.

Future Improvements:

- Encrypt the password and store it securely.

- Use a more advanced interface for user input (like a keypad or touchscreen).

- Add buzzer or LCD for better feedback.

- Support for variable-length passwords.